🚀 SPRINT 2 — FULL DELIVERABLES

Priorities 1 & 2 are CRITICAL. Delivered first. No scaffolds. No placeholders.

scripts/ndr-high-cardinality.zeek — PRIORITY 1 [CRITICAL] — FULL LOGIC

##############################################################

# NDR High-Cardinality Connection Tracker — Engineer 1

# Blueprint v1.2 | Sprint 2 | Priority 1 [CRITICAL]

# Modification 1 Response: SCAFFOLDED → FULL IMPLEMENTATION

#

# Logic: Track unique destination IPs per source host

# Window 1: 5 minutes  → threshold > 15 unique dst IPs = ALERT

# Window 2: 60 minutes → threshold > 15 unique dst IPs = ALERT

# Rationale: Detects port scans, lateral movement, C2 fan-out

# ECS Output: zeek.ndr_high_cardinality | event.kind = alert

##############################################################

module NDR_HighCardinality;

export {

redef enum Log::ID += { LOG };

type HighCardRecord: record {

# ECS: @timestamp

ts:               time    &log;

# ECS: source.ip

src_ip:           addr    &log;

# ECS: network.*

unique_dst_count: count   &log;

unique_dst_ports: count   &log;

# Detection window context

window_seconds:   count   &log;

sample_dst_ips:   set[addr] &log;

# ECS: event.*

event_kind:       string  &log &default="alert";

event_category:   string  &log &default="intrusion_detection";

event_type:       string  &log &default="indicator";

event_dataset:    string  &log &default="zeek.ndr_high_cardinality";

# Triage Agent fields

risk_score:       double  &log;

detection_class:  string  &log;   # lateral_movement | port_scan | c2_fanout

# Blueprint labels

blueprint_version: string &log &default="v1.2";

tier:             string  &log &default="tier1_behavioral";

};

global log_high_cardinality: event(rec: HighCardRecord);

# Architect-mandated thresholds

const UNIQUE_DST_THRESHOLD    = 15    &redef;   # > 15 unique dst IPs

const WINDOW_5MIN_SECS        = 300   &redef;

const WINDOW_60MIN_SECS       = 3600  &redef;

const MAX_SAMPLE_IPS          = 10    &redef;   # Cap sample set for log verbosity

}

# ─── STATE TABLES ────────────────────────────────────────────

# 5-minute rolling window: src_ip → set of dst_ips seen

global tracker_5min: table[addr] of set[addr]

&create_expire=5mins

&expire_func=check_5min_expiry;

# 60-minute rolling window

global tracker_60min: table[addr] of set[addr]

&create_expire=60mins

&expire_func=check_60min_expiry;

# Port tracking per source (for port scan classification)

global port_tracker_5min: table[addr] of set[port]

&create_expire=5mins;

# Dedup: prevent re-alerting same src within cooldown

global alert_cooldown: table[addr, count] of time

&create_expire=5mins;

# ─── LOG STREAM INIT ─────────────────────────────────────────

event zeek_init()

{

Log::create_stream(NDR_HighCardinality::LOG,

[$columns=HighCardRecord,

$path="ndr_high_cardinality"]);

}

# ─── ALERT EMISSION FUNCTION ─────────────────────────────────

function emit_alert(src: addr, dst_set: set[addr],

port_set: set[port], window: count)

{

local cooldown_key = [src, window];

if(cooldown_key in alert_cooldown) return;

alert_cooldown[cooldown_key] = network_time();

# Classify detection type

local dclass = "high_cardinality";

if(|port_set| > 50)

dclass = "port_scan";

else if(|dst_set| > 30 && |port_set| <= 5)

dclass = "c2_fanout";

else if(|dst_set| > 15 && Site::is_local_addr(src))

dclass = "lateral_movement";

# Risk score: scales with cardinality, penalizes internal src

local base_score = (|dst_set| / 100.0) * 100.0;

if(base_score > 100.0) base_score = 100.0;

if(Site::is_local_addr(src)) base_score = base_score * 1.2;

if(base_score > 100.0) base_score = 100.0;

# Build sample IP set (capped)

local sample: set[addr] = set();

local count_added = 0;

for(d in dst_set) {

if(count_added >= MAX_SAMPLE_IPS) break;

add sample[d];

++count_added;

}

local rec = HighCardRecord(

$ts               = network_time(),

$src_ip           = src,

$unique_dst_count = |dst_set|,

$unique_dst_ports = |port_set|,

$window_seconds   = window,

$sample_dst_ips   = sample,

$risk_score       = base_score,

$detection_class  = dclass

);

Log::write(NDR_HighCardinality::LOG, rec);

}

# ─── CORE EVENT HOOK ─────────────────────────────────────────

event connection_established(c: connection)

{

local src  = c$id$orig_h;

local dst  = c$id$resp_h;

local dport = c$id$resp_p;

# ── 5-MINUTE WINDOW TRACKING ──────────────────────────────

if(src !in tracker_5min)

tracker_5min[src] = set();

add tracker_5min[src][dst];

if(src !in port_tracker_5min)

port_tracker_5min[src] = set();

add port_tracker_5min[src][dport];

if(|tracker_5min[src]| > UNIQUE_DST_THRESHOLD) {

local ports_5 = src in port_tracker_5min

? port_tracker_5min[src] : set();

emit_alert(src, tracker_5min[src], ports_5,

WINDOW_5MIN_SECS);

}

# ── 60-MINUTE WINDOW TRACKING ─────────────────────────────

if(src !in tracker_60min)

tracker_60min[src] = set();

add tracker_60min[src][dst];

if(|tracker_60min[src]| > UNIQUE_DST_THRESHOLD) {

local ports_60 = src in port_tracker_5min

? port_tracker_5min[src] : set();

emit_alert(src, tracker_60min[src], ports_60,

WINDOW_60MIN_SECS);

}

}

# ─── EXPIRY CALLBACKS (clean emit on window close) ───────────

function check_5min_expiry(t: table[addr] of set[addr], idx: addr): interval

{

if(idx in t && |t[idx]| > UNIQUE_DST_THRESHOLD) {

local ports = idx in port_tracker_5min

? port_tracker_5min[idx] : set();

emit_alert(idx, t[idx], ports, WINDOW_5MIN_SECS);

}

return 0secs;

}

function check_60min_expiry(t: table[addr] of set[addr], idx: addr): interval

{

if(idx in t && |t[idx]| > UNIQUE_DST_THRESHOLD) {

emit_alert(idx, t[idx], set(), WINDOW_60MIN_SECS);

}

return 0secs;

}

scripts/ndr-heartbeat.zeek — PRIORITY 2 [CRITICAL] — TELEMETRY HEARTBEAT

##############################################################

# NDR Telemetry Heartbeat — Engineer 1

# Blueprint v1.2 | Sprint 2 | Priority 2 [CRITICAL]

# SPOF Response: Eliminates silent sensor failure blind spot

#

# Mechanism:

#   - Zeek emits synthetic heartbeat event every 60 seconds

#   - n8n monitors heartbeat log; no receipt in 90s = SENSOR DARK

#   - Auto-restart directive is embedded in alert payload

#   - Downstream: Conductor / Triage Agent receive 'sensor_dark' event

##############################################################

module NDR_Heartbeat;

export {

redef enum Log::ID += { LOG };

type HeartbeatRecord: record {

# ECS: @timestamp

ts:               time    &log;

# Heartbeat identity

sensor_id:        string  &log;

sensor_host:      string  &log;

sequence_number:  count   &log;

# ECS: event.*

event_kind:       string  &log;

event_dataset:    string  &log &default="zeek.ndr_heartbeat";

event_category:   string  &log &default="process";

# Health metrics

status:           string  &log;   # ALIVE | DEGRADED | DARK

uptime_seconds:   double  &log;

packets_processed: count  &log &optional;

# Auto-restart directive (consumed by n8n watchdog)

auto_restart_eligible: bool &log &default=T;

restart_command:  string  &log

&default="/opt/zeek/bin/zeekctl restart";

# Blueprint labels

blueprint_version: string &log &default="v1.2";

tier:             string  &log &default="infrastructure";

};

# Configurable intervals — Architect mandated: 60s emit / 90s timeout

const HEARTBEAT_INTERVAL  = 60secs  &redef;

const SENSOR_DARK_TIMEOUT = 90secs  &redef;

global log_heartbeat: event(rec: HeartbeatRecord);

}

# ─── STATE ───────────────────────────────────────────────────

global hb_sequence:   count  = 0;

global hb_start_time: time;

global hb_packet_count: count = 0;

# ─── LOG STREAM INIT ─────────────────────────────────────────

event zeek_init()

{

Log::create_stream(NDR_Heartbeat::LOG,

[$columns=HeartbeatRecord,

$path="ndr_heartbeat"]);

hb_start_time = network_time();

# Schedule first heartbeat

schedule HEARTBEAT_INTERVAL { NDR_Heartbeat::emit_heartbeat() };

}

# ─── HEARTBEAT EMISSION ──────────────────────────────────────

event NDR_Heartbeat::emit_heartbeat()

{

++hb_sequence;

local uptime = interval_to_double(

network_time() - hb_start_time);

local rec = HeartbeatRecord(

$ts               = network_time(),

$sensor_id        = fmt("zeek-sensor-%s",

gethostname()),

$sensor_host      = gethostname(),

$sequence_number  = hb_sequence,

$event_kind       = "event",

$status           = "ALIVE",

$uptime_seconds   = uptime,

$packets_processed = hb_packet_count

);

Log::write(NDR_Heartbeat::LOG, rec);

# Reschedule — perpetual loop

schedule HEARTBEAT_INTERVAL { NDR_Heartbeat::emit_heartbeat() };

}

# ─── PACKET COUNTER (health metric) ─────────────────────────

event new_packet(c: connection, p: pkt_hdr)

{

++hb_packet_count;

}

# ─── GRACEFUL SHUTDOWN HEARTBEAT ─────────────────────────────

event zeek_done()

{

local rec = HeartbeatRecord(

$ts               = network_time(),

$sensor_id        = fmt("zeek-sensor-%s",

gethostname()),

$sensor_host      = gethostname(),

$sequence_number  = hb_sequence + 1,

$event_kind       = "alert",

$status           = "DARK",

$uptime_seconds   = interval_to_double(

network_time() - hb_start_time),

$auto_restart_eligible = T

);

Log::write(NDR_Heartbeat::LOG, rec);

}